## EXP-1: ARDUINO UNO-BASED SMART HOME SAFETY AND OCCUPANCY MONITORING SYSTEM USING GAS, PIR AND ULTRASONIC SENSORS

# CODE:

#define GAS_PIN A0

#define PIR_PIN 2

#define TRIG_PIN 8

#define ECHO_PIN 9

#define BUZZER 10

#define LED 11

void setup()

{

Serial.begin(9600);

pinMode(PIR_PIN, INPUT);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER, OUTPUT);

pinMode(LED, OUTPUT);

}

void loop()

{

int gas = analogRead(GAS_PIN);

int motion = digitalRead(PIR_PIN);

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH);

float distance = duration * 0.0343 / 2;

if (gas > 500)

digitalWrite(BUZZER, HIGH);

else

digitalWrite(BUZZER, LOW);

digitalWrite(LED, motion);

Serial.print("Gas Level: ");

Serial.println(gas);

Serial.print("Motion: ");

if (motion)

Serial.println("DETECTED");

else

Serial.println("NOT DETECTED");

Serial.print("Distance: ");

Serial.print(distance, 1);

Serial.println(" cm");

if (gas > 500)

Serial.println("GAS ALERT!");

else if (motion)

Serial.println("OCCUPANCY DETECTED");

else

Serial.println("SYSTEM SAFE");

Serial.println("--------------------");

delay(1000);

}

# OUTPUT:

# REFLECTION: I learned how to interface gas, PIR, and ultrasonic sensors with Arduino UNO for smart home safety monitoring. I understood how to detect gas leakage, human motion, and object distance using Embedded C programming. This experiment improved my knowledge of sensor interfacing, alert systems, and Proteus simulation.